Reacting systems: Use of Matlab as a problem solver tool.

Prepared for

Reacting Systems.

Summer 1999

Dr. Luis H. Garcia-Rubio.

Prepared by.

Omega-Chi-Epsilon.

Chemical Engineer’s Honors Society.

Andres M. Cardenas-Valencia.

Contents:

- Introduction to Matlab. Help

- Script files, the subroutine and function structure.

- Problem 9.4 Fogler 2nd edition. Non linear equation solver.

- Problem 4.26 Fogler 2nd edition. Differential equation

- Problem 8.6 and 8.7  Fogler 2nd edition. System of differential equations.

Chemical Engineering Department

University of South Florida.

Problem 9.4 in Fogler 2nd. Edition.

A mixture of 50% A and 50% B is charged at constant volume batch reactor in which equilibrium is rapidly achieved. The initial total concentration is 3.0 mol/dm3.

- Calculate the equilibrium concentrations and conversion of A at 330 K for the reaction sequence:

Reaction 1: A+B⇔C+D Kc1(330K)=4.0   Kc1(350K)=2.63

Reaction 2: C+B⇔X+Y Kc2(330K)=1.0   Kc2(350K)=1.51

- Suppose now the temperature is increased to 350 K. As a result a third reaction must now be considered in addition to the above 2 reactions.

Reaction 3: A+X⇔Z Kc3(350K)=5.0dm3mol

Calculate the equilibrium concentrations, conversion of A and selectivities for the pair of reagents.

a) at T=330. The equations to solve are:

Kc1(330K)=4.0=Cc∗CDCA∗CB

Kc2(330K)=1.0=Cx∗CYCC∗CB

The stoichiometric relations stated in the equations let us write:

CA=CA0(1−X1), where X1 is the conversion of reaction 1

B and C are involved in two of the other reactions so the conversion for both will appear:

CB=CB0(1−X1−X2): NOTE CA0=CB0

CC=CA0(X1−X2)

CD=CA0X1CX=CA0X2CY=CA0X2

Now, the implementation in MATLAB:

First of all, For non linear algebraic equations the subroutine to use in matlab is called fsolve. We type help fsolve in the command window.

- We need to create a function that retrieves the value of the equations equated to zero we want to solve. This is the ‘F’ they’re referring to in the help.

- To do so, the word function is required at the beginning of the file. Then the output of the function and the in parenthesis the inputs:

The function I created is called prob94.m and the input parameters is the vector x. This vector x corresponds to the different conversions and concentrations as follows:

x=[x(1)x(2)x(3)x(4)x(5)x(6)x(7)x(8)]

x=[x1x2 CA CB CC CD CX CY]

The file that I have created is as follows:

function F=prob94(x)

F=[4-x(5)*x(6)/(x(3)*x(4))

1-x(7)*x(8)/(x(5)*x(4))

x(3)-1.5*(1-x(1))

x(4)-1.5*(1-x(1)-x(2))

x(5)-1.5*(x(1)-x(2))

x(6)-1.5*x(1)

x(7)-1.5*x(2)

x(8)-1.5*x(2)];

Then from the command window we can give matlab the instructions to solve the set of equations.

- First make sure that you are in the right subdirectory

- Type the command line and if you want save the answer in a variable verify them.

- x = fsolve('prob94',.5*ones(1,8))

b) In this case the three equations have to be considered. Now, the problem is slightly more complicated why?????????????

Problem 4.26 in Fogler 2nd. Edition.

A catalyst is a material that affects the rate of a chemical reaction yet emerges unchanged from the reaction. For example, consider the reaction

A+B→C+BIn which

rA=-kCACB

Where B is the catalyst. The reaction is taking place in a semibatch reactor, in which 100 ft3 of a solution containing 2 lbmol/ft3 of A is initially present. No B is present initially. Starting at time t=0.5 ft3/min of a solution containing 0.5 lbmol/ft3 of B is fed into the reactor. The reactor is isothermal and k= 0.2 ft3/lbmolmin

- How many moles are present in the reactor after half an hour

- Plot the conversion as a function of time.

The solution of the problem is given now:

Now, the implementation in MATLAB for part c:

First of all, for differential equations solver the subroutine to use in matlab is called ode23 (low order Runge-kutta method). We type again help ode23 to find out the format of the inputs.

- We need to create a function that retrieves the value of the derivatives of the independent variables to solve with respect to the independent variable. This has to be a column vector. This is the ‘F’ they’re referring to in the help.

- To do so, the word function is required at the beginning of the file. Then the output of the function and the in parenthesis the inputs which in this case are t (independent variable and conversion x, (dependent variable).

The function I created is called prob426.m  Notice in this case the input parameters is the scalar x. BUT we could and we will use vectors too

The file that I have created is as follows:

function dX=prob426(t,X)

dX=[(0.02*t-10*X)/(100+5*t)];

Now from the command window, we give the instructions for the equation to be solved.

- Again, first make sure that you are in the right subdirectory

- Type the command line and if you want save the answer in a variable write them.

- [t,x]=ode23('prob426',[0:50:750],0)

The command scripts and the answers for problems 8.6 and 8.7 are given next:

Example solved, 8.6)

function main=ex86

%This are the limiting catalyst weights

W0=0;

WF=50;

%This is the initial condition

X0=[0 450 1];

[W,X]=ode23('ex86b',W0,WF,X0);

whitebg

subplot(3,1,1),plot(W,X(:,1));

%and now for T

subplot(3,1,2),plot(W,X(:,2));

%and now for Dimensionless pressure

subplot(3,1,3),plot(W,X(:,3));

The function the retrieves the derivatives is

function rate=ex86a(W,X)

Cao=.271;

Fao=5.42;

alpha=.019;

k=.133.*exp(3776.66.*(1/450-1./(X(2))));

Ca=Cao.*(1-X(1)).*450.*(X(3))./((1+X(1)).*(X(2)));

rate=[(k.*Ca)./Fao

20000.*k.*Ca./(Fao.*40)

(-X(1)-1).*X(2).*alpha./(450.*2.*X(3))];

Problem 8.7) The command script file is

function main=ex87

%This are the limiting catalyst weights

W0=0;

WF=37;

%This is the initial condition

X0=[0 450 1];

[W,X]=ode23('ex87a',W0,WF,X0);

whitebg

subplot(3,1,1),plot(W,X(:,1));

%and now for T

subplot(3,1,2),plot(W,X(:,2));

%and now for Dimensionless pressure

subplot(3,1,3),plot(W,X(:,3));

Now the auxiliary function that retrieves the derivative is the next function:

function rate=ex87a(W,X)

Cao=.271;

Fao=5.42;

alpha=.019;

k=.133.*exp(3776.66.*(1/450-1./(X(2))));

Ca=Cao.*(1-X(1)).*450.*(X(3))./((1+X(1)).*(X(2)));

rate=[(k.*Ca)./Fao

20000.*k.*Ca./(Fao.*40)

(-X(1)-1).*X(2).*alpha./(450.*2.*X(3))];